uOttawa UNIVERSITÉ D'OTTAWA · UNIVERSITY OF OTTAWA

# PDR — EOSAT-1 Multispectral Imager

Preliminary Design Review

SpaceCDF

2026

Faculty of Engineering · School of Engineering Design and Teaching Innovation (SEDTI)

SCDF-PDR-001 · 6U EO CubeSat · Issue 1.0 · 2026-05-12

Classification: Internal · Demo

# Document Information

| Document code | SCDF-PDR-001 |
| --- | --- |
| Title | Preliminary Design Review — EOSAT-1 Multispectral Imager |
| Study / Mission | 6U EO CubeSat |
| Issue | 1.0 |
| Date | 2026-05-12 |
| Prepared by | SpaceCDF AI Concurrent Design Facility |
| Reviewed by | [TBD] |
| Approved by | [TBD] |
| Classification | Internal · Demo |
| Applies to | EOSAT-1 Multispectral Imager |

# Change Record

| Issue | Date | By | Summary of changes |
| --- | --- | --- | --- |
| 1.0 | 2026-05-12 | SpaceCDF | Initial PDR issue generated from converged design loop. |

# Acknowledgement — Generative AI (AIG)

This document was produced with the assistance of generative AI as part of the SpaceCDF Concurrent Design Facility workflow.  The design-loop convergence, agent rationales, embedded figures and document rendering are generated by the SpaceCDF backend (Python · matplotlib · python-docx) guided by ECSS, NASA SEH and SMAD-4 references.  Editorial framing, worked examples and pedagogical commentary remain owned by the SpaceCDF teaching team.

Attribution follows the AIG (Assisted by Generative AI) framework — Peters (2023), Logos IA-EN, CC BY-NC-SA 4.0 — https://mpeters.uqo.ca/en/logos-ia-en-peters-2023/

Any course deliverable that incorporates content from this document, or from any document exported by SpaceCDF, must in turn carry the AIG badge and a short note describing how generative AI was used.

# Acronyms & Abbreviations

| Acronym | Definition |
| --- | --- |
| AIT | Assembly, Integration and Test |
| AOCS | Attitude and Orbit Control Subsystem |
| BoL | Beginning of Life |
| CDF | Concurrent Design Facility |
| CDR | Critical Design Review |
| ConOps | Concept of Operations |
| DDT&E | Design, Development, Test and Evaluation |
| EPS | Electrical Power Subsystem |
| EoL | End of Life |
| FMECA | Failure Mode, Effects and Criticality Analysis |
| GS | Ground Station |
| IADC | Inter-Agency Space Debris Coordination Committee |
| ITU | International Telecommunication Union |
| MLI | Multi-Layer Insulation |
| MRD | Mission Requirements Document |
| MTBF | Mean Time Between Failures |
| OBDH | On-Board Data Handling |
| PDR | Preliminary Design Review |
| RPN | Risk Priority Number |
| RW | Reaction Wheel |
| SA | Solar Array |
| SCDF | SpaceCDF Concurrent Design Facility |
| SRR | System Requirements Review |
| SSO | Sun-Synchronous Orbit |
| TCS | Thermal Control Subsystem |
| TRL | Technology Readiness Level |
| TT&C | Telemetry, Tracking and Command |
| VP | Verification Plan |
| WBS | Work Breakdown Structure |

# Applicable Documents

| ID | Reference | Title |
| --- | --- | --- |
| AD-01 | ECSS-M-ST-10C Rev.1 | Space project management |
| AD-02 | ECSS-E-ST-10C Rev.1 | System engineering general requirements |
| AD-03 | ECSS-E-ST-10-02C | Verification requirements |
| AD-04 | ECSS-E-ST-10-06C | Technical requirements specification |
| AD-05 | ECSS-Q-ST-30C | Dependability |
| AD-06 | ECSS-Q-ST-40C Rev.1 | Safety |

# Reference Documents

| ID | Reference | Title |
| --- | --- | --- |
| RD-01 | NASA SP-2016-6105 | NASA Systems Engineering Handbook Rev. 2 |
| RD-02 | NPR 7123.1D | NASA Systems Engineering Processes and Requirements |
| RD-03 | SMAD-4 | Space Mission Analysis and Design (4th ed.) |
| RD-04 | ECSS-U-AS-10C Rev.2 | Space debris mitigation requirements |
| RD-05 | ISO 24113:2023 | Space debris mitigation requirements |
| RD-06 | ITU-R P.618-13 | Propagation data for Earth-space links |
| RD-07 | ITU-R P.676-13 | Attenuation by atmospheric gases |

# Table of Contents

Right-click and select 'Update Field' to populate.

# 1. Scope

## 1.1 Purpose

This document is the Preliminary Design Review for the EOSAT-1 Multispectral Imager.  It captures the converged engineering baseline produced by the SpaceCDF AI Concurrent Design Facility and provides the evidence required for the PDR review gate per ECSS-M-ST-10C Rev.1.

## 1.2 Applicability

The contents are applicable to all engineering disciplines, project management, product assurance and operations.  Subsystem-level data items inherit applicability from the parent applicable documents listed above.

## 1.3 Conventions

All units are SI unless otherwise stated.  Margins are quoted relative to the nominal value (i.e. with-margin = nominal × (1 + margin%)).  Status colours follow the SpaceCDF / ECSS convention: green = compliant, amber = marginal, red = non-compliant or exceeded.

# 2. Mission Overview

Mission 'EOSAT-1 Multispectral Imager' is a Earth Observation mission carried out by 1 nano-class spacecraft over a design life of 3.0 years.  Top-level mission targets are 12.0 kg launch mass and 5.0 MEUR total programme cost.

| Mission attribute | Value |
| --- | --- |
| Mission name | EOSAT-1 Multispectral Imager |
| Mission type | Earth Observation |
| Spacecraft class | nano |
| Number of spacecraft | 1 |
| Design lifetime (yr) | 3.00 |
| Reliability target | 0.90 |
| Cost target (MEUR) | 5.00 |
| Mass target (kg) | 12.00 |
| Ground stations | KSAT Svalbard, KSAT Troll |

Table 1 — Top-level mission descriptor.

## 2.1 Payload complement

| Name | Type | Mass (kg) | Power (W) | Rate (Mbps) | Pointing (°) | Duty |
| --- | --- | --- | --- | --- | --- | --- |
| Multispectral Imager | optical_imager | 1.50 | 20.0 | 100.0 | 0.100 | 25% |

Table 2 — Payload complement.

# 3. Concept of Operations & Orbit

| Parameter | Value |
| --- | --- |
| Orbit type | SSO |
| Altitude (km) | 450 |
| Inclination (deg) | 97.40 |
| Period (min) | 93.4 |
| Orbits per day | 15.41 |
| Velocity (m/s) | 7644.4 |
| Eclipse fraction | 0.384 |
| Footprint radius (km) | 2327 |
| Contact time / day (s) | 2055 |
| Mission duration (yr) | 3.0 |

Table 3 — Orbital parameters.

Figure 1 — SSO orbit geometry with shaded eclipse arc.

Figure 2 — Indicative ground track over three consecutive orbits.

### Rationale

Orbit chosen as SSO at 450 km altitude and inclination 97.40°.  The Keplerian period is 93.4 min (15.4 orbits / day) with an eclipse fraction of 0.38 (cylindrical shadow).  A high-latitude ground station near 78° provides ≈34.2 min of contact / day.

### Assumptions

Cylindrical Earth-shadow eclipse model (no penumbra).

Ground-station latitude assumed 78° (Svalbard-class).

Station-keeping ΔV from JR-1971 drag model over 3-yr life.

## 3.1 ΔV breakdown

| Component | ΔV (m/s) | Rationale |
| --- | --- | --- |
| Station-keeping | 4.6 | Drag makeup over 3 yr design life. |
| Collision avoidance | 3.0 | Allowance of ~1 m/s per year for collision-avoidance burns. |
| Deorbit / EOL | 71.7 | Lower perigee for ≤25-yr re-entry (IADC compliance). |

Table 4 — ΔV budget breakdown.

# 4. System Architecture

The spacecraft system architecture follows the canonical CubeSat-class topology with a payload routing data through the OBDH to the TT&C subsystem, while the EPS distributes regulated power to all platform subsystems.  AOCS drives propulsion (when fitted) and the thermal subsystem couples conductively to the structure.

Figure 3 — Spacecraft system architecture — block diagram.

# 5. Payload

## 5.1 Multispectral Imager

4-band ocean colour imager (443, 560, 665, 865 nm)

| Attribute | Value |
| --- | --- |
| Type | optical_imager |
| Mass (kg) | 1.50 |
| Power, mean (W) | 20.0 |
| Power, peak (W) | 45.0 |
| Data rate (Mbps) | 100.00 |
| Data volume / day (GB) | 2.00 |
| Pointing accuracy (°) | 0.100 |
| Duty cycle (%) | 25 |

# 6. Electrical Power Subsystem

### Rationale

Electrical power system sized for a worst-case sunlight load of 31.0 W and worst-case eclipse load of 36.0 W.  Solar array delivers 76.5 W BoL / 70.9 W EoL after 3.0 yr of degradation, covering 0.216 m² of 29.5%-efficient cells.  Battery capacity 71.7 Wh chosen for 30.0% depth-of-discharge over the eclipse duration.

### Assumptions

Solar cell efficiency 29.5% (class-appropriate triple-junction GaAs / KB-resolved cell).

Specific power 80 W/kg.

Mode profile estimated from payload duty cycles.

Heater allocation 10.0 W during eclipse.

Battery DoD limit 30% (ECSS Li-ion practice).

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| Battery Capacity | 71.7 | Wh | 20% |
| Battery DOD | 30 | % | 20% |
| Battery Mass | 0.48 | kg | 20% |
| EPS Cost | 12 | kEUR | 20% |
| EPS Total Mass | 1.57 | kg | 20% |
| Solar Array Area | 0.216 | m² | 20% |
| Solar Array Mass | 0.89 | kg | 20% |
| SA Power BOL | 76.5 | W | 20% |
| SA Power EOL | 70.9 | W | 20% |
| Total Power (Eclipse) | 36 | W | 20% |
| Total Power (Sunlight) | 31 | W | 20% |

Figure 4 — Orbit-averaged power profile (sunlit / eclipse, SA EoL line).

## 6.2 Battery

| Parameter | Value |
| --- | --- |
| Capacity (Wh) | 71.7 |
| Depth of discharge (%) | 30.0 |
| Mass (kg) | 0.48 |
| Chemistry | Li-ion (assumed) |

# 7. Thermal Control Subsystem

### Rationale

Thermal control sized for a hot-case internal dissipation of 31.0 W on a 0.22 m² external surface (cubesat).  The radiator area is 0.113 m² (ε=0.85, α=0.15) and rejects to space at T_max=50 °C.  In the cold case (eclipse, 30 % standby load) heaters supply 0.0 W to hold the platform above T_min=-20 °C.

### Assumptions

Stefan–Boltzmann steady-state energy balance, no transient analysis.

Solar flux 1361 W/m², Earth IR 237 W/m², albedo 0.30.

Radiator: ε=0.85, α=0.15 (white paint / OSR).

MLI effective emissivity 0.01 (15-layer blanket).

Platform allowable range −20 °C to +50 °C; payload limits set separately.

### Warnings & open issues

Radiator area 0.11m² exceeds 50% of S/C area — consider active cooling

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| Eclipse Heater Power | 0 | W | 20% |
| Radiator Area | 0.113 | m² | 20% |
| Radiator Mass | 0 | kg | 20% |
| TCS Cost | 5 | kEUR | 20% |
| TCS Total Mass | 0.35 | kg | 20% |

Figure 5 — Hot and cold case temperatures across the principal thermal nodes.

| Node | Cold (°C) | Hot (°C) | Min limit | Max limit |
| --- | --- | --- | --- | --- |
| Spacecraft bus | -20.0 | 50.0 | -20.0 | 50.0 |
| Battery | 2.0 | 45.0 | 0.0 | 35.0 |
| Payload optics | -20.0 | 40.0 | -30.0 | 40.0 |
| OBDH/avionics | -15.0 | 47.0 | -40.0 | 70.0 |

## 7.1 Surface treatments

| Surface | α | ε | Area (m²) |
| --- | --- | --- | --- |
| Radiator (OSR) | 0.15 | 0.85 | 0.113 |
| MLI (15-layer) | 0.10 | 0.01 | 0.107 |

# 8. AOCS

### Rationale

AOCS sized for a pointing requirement of 0.100° around a earth central body.  Disturbance torque budget is dominated by gravity gradient (9.94e-09 Nm), with solar pressure (6.13e-09), aerodynamic (2.20e-08) and magnetic (2.44e-06) contributions over one orbit summing to 2.48e-06 Nm.  Reaction wheel momentum capacity 0.003 Nms provides one-orbit absorption before desaturation.

### Assumptions

Spacecraft maximum dimension 0.30 m (class-typical).

Wet area assumed 0.09 m² (cubical bus).

Worst-case attitude offsets: 5° from nadir for gravity gradient; 0.05 m solar CoP offset; 0.05 m aero CoP offset.

Magnetorquer dipole desaturation effective at mid-latitudes (Earth orbits only).

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| AOCS Cost | 204 | kEUR | 20% |
| AOCS Mass | 2.04 | kg | 20% |
| Pointing Accuracy | 0.1 | deg | 20% |
| AOCS Power | 15 | W | 20% |
| Aerodynamic Torque | 0 | Nm | 20% |
| Gravity Gradient Torque | 0 | Nm | 20% |
| Magnetic Torque | 2e-06 | Nm | 20% |
| Solar Pressure Torque | 0 | Nm | 20% |
| Total Disturbance Torque | 2e-06 | Nm | 20% |
| Wheel Momentum | 0.003 | Nms | 20% |

## 8.1 Disturbance torque budget

| Source | Torque (Nm) |
| --- | --- |
| Gravity gradient | 9.94e-09 |
| Solar pressure | 6.13e-09 |
| Aerodynamic | 2.20e-08 |
| Magnetic | 2.44e-06 |

## 8.2 Pointing error budget

| Contributor | 1σ (arcsec) |
| --- | --- |
| Sensor noise (star tracker) | 10.0 |
| Actuator quantisation (RW) | 30.0 |
| Disturbance reject. residual | 60.0 |
| Alignment | 36.0 |
| Achieved (RSS) | 76.8 |

# 9. Propulsion

### Rationale

No propulsion required — the spacecraft is a nano class at 450 km altitude, where natural atmospheric drag completes deorbit within the IADC 25-year window without active manoeuvres.

### Assumptions

Natural-decay regime (low-LEO nano/micro-class).

No collision-avoidance budget; mission-dependent JCA on-orbit.

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| Propulsion Cost | 0 | kEUR | 20% |
| Total Delta-V | 0 | m/s | 20% |
| Specific Impulse | 0 | s | 20% |
| Propellant Mass | 0 | kg | 20% |
| Total Impulse | 0 | Ns | 20% |
| Propulsion System Mass | 0 | kg | 20% |
| Propulsion Type | none |  | 20% |

# 10. Telemetry, Tracking & Command

### Rationale

Communications architecture is a dual link: an always-on UHF TT&C channel for command and housekeeping (margin 29.4 dB) and a higher-rate X payload downlink at 8.2 GHz (margin 7.7 dB).  EIRP is 8.0 dBW; free-space loss is 176.5 dB at the worst-case slant range of 1944 km.

### Assumptions

5° minimum elevation; ITU-R P.676/P.618 atmospheric model.

GS antenna 13.0 m, efficiency 0.55, system noise temperature 200 K.

Modulation coding rate 0.5, protocol overhead 10%, implementation loss 2 dB.

Required Eb/N0 10 dB for BER 10⁻⁶ at the assumed modulation.

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| Data Downlinked/Day | 2.4 | GB | 20% |
| Downlink Margin (worst) | 7.7 | dB | 20% |
| Downlink Data Rate | 9.34e+06 | bps | 20% |
| Payload DL Margin | 7.7 | dB | 20% |
| TTC Cost | 32 | kEUR | 20% |
| TTC Margin | 29.4 | dB | 20% |
| TTC Mass | 0.42 | kg | 10% |
| TTC Power | 11 | W | 20% |
| Uplink Margin | 39.4 | dB | 20% |

Figure 6 — Payload downlink link budget waterfall (positive = gain, negative = loss).

| Term | Δ (dB) |
| --- | --- |
| Tx power (dBW) | +3.01 |
| Tx antenna gain | +6.00 |
| Tx line loss | -1.00 |
| Free-space loss | -176.50 |
| Atmospheric gas | -1.14 |
| Rain | -0.00 |
| Polarisation | -0.50 |
| Pointing | -1.00 |
| GS antenna gain | +58.37 |
| Boltzmann (k) | +228.60 |
| System noise T | -23.01 |
| Data rate | -69.71 |
| Required Eb/N0 | -10.00 |
| Implementation loss | -2.00 |

## 10.1 Link summary

| Parameter | Value |
| --- | --- |
| Band | X |
| Frequency (GHz) | 8.200 |
| S/C Tx power (W) | 2.0 |
| S/C antenna gain (dBi) | 6.0 |
| GS antenna Ø (m) | 13.00 |
| GS antenna gain (dBi) | 58.4 |
| Slant range (km) | 1944 |
| User data rate (bps) | 9,344,907 |
| Data downlinked / day (GB) | 2.40 |
| Downlink margin (dB) | 7.7 |
| TT&C margin (dB) | 29.4 |
| Uplink margin (dB) | 39.4 |

# 11. Data Handling

### Rationale

Data architecture sized for 2.0 GB/day generated (2.4 GB/day downlinked).  Onboard storage 4.0 GB provides 2-orbit redundancy.  OBDH mass 0.16 kg from class factor × storage demand.

### Assumptions

2:1 lossless compression when payload data rate is used as input (explicit per-day volume overrides this).

Storage = 2 × daily generation (operational buffer).

OBDH mass scales with class baseline × (1 + storage/100 GB).

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| Data Downlinked/Day | 2.4 | GB | 20% |
| Data Generated/Day | 2 | GB | 20% |
| OBDH Cost | 16 | kEUR | 20% |
| OBDH Mass | 0.16 | kg | 10% |
| Storage Required | 4 | GB | 20% |

# 12. Structure & Mechanisms

### Rationale

Structure mass is taken from the heritage class-fraction database with Phase-A margin of 20%.  Detailed FEM, modal analysis, and mechanism qualification are deferred to PDR/CDR.

| Parameter | Value | Unit | Margin |
| --- | --- | --- | --- |
| Structure Cost | 8 | kEUR | 20% |
| Structure Fraction | 0.141 |  | 20% |
| Structure Mass | 0.99 | kg | 20% |

# 13. System Budgets

Figure 7 — Normalised share of each system budget by subsystem.

## 13.1 Mass budget

Figure 8 — Mass budget contributions and total (kg).

| Subsystem | Equipment | Nominal (kg) | Margin (%) | With margin (kg) |
| --- | --- | --- | --- | --- |
| payload | Payload Multispectral Imager Mass | 1.5 | 10 | 1.65 |
| aocs | AOCS Mass | 2.04 | 20 | 2.45 |
| structure | Structure Mass | 0.99 | 20 | 1.19 |
| TOTAL |  | 4.53 | 56 | 5.29 |

Allocation: 12.00 kg — margin 55.9% — status GREEN

## 13.2 Power budget

Figure 9 — Power budget contributions and total (W).

| Subsystem | Equipment | Nominal (W) | Margin (%) | With margin (W) |
| --- | --- | --- | --- | --- |
| payload | Payload Multispectral Imager Power | 20 | 10 | 22 |
| aocs | AOCS Power | 15 | 20 | 18 |
| TOTAL |  | 35 | 44 | 40 |

Allocation: 70.90 W — margin 43.6% — status GREEN

## 13.4 Cost budget

Figure 10 — Cost budget contributions and total (kEUR).

| Subsystem | Equipment | Nominal (kEUR) | Margin (%) | With margin (kEUR) |
| --- | --- | --- | --- | --- |
| aocs | AOCS Cost | 204 | 10 | 224 |
| propulsion | Propulsion Cost | 0 | 10 | 0 |
| structure | Structure Cost | 8 | 10 | 8.8 |
| TOTAL |  | 212 | 95 | 233 |

Allocation: 5.00 MEUR — margin 95.3% — status GREEN

# 14. Equipment List

No KB-selected equipment recorded in the design state.

# 15. Compliance & Verification

Figure 11 — Compliance status across all requirements.

| Req ID | Description | Achieved | Margin % | Status |
| --- | --- | --- | --- | --- |
| REQ-SYS-001 | The spacecraft shall have a design lifetime of at least 3.0 years | 3.00 | 0.0 | marginal |
| REQ-SYS-002 | The spacecraft wet mass shall not exceed 12.0 kg | 7.03 | 41.4 | compliant |
| REQ-SYS-003 | The total mission cost shall not exceed 5.0 MEUR | 0.70 | 86.0 | compliant |
| REQ-AOCS-001 | The spacecraft shall provide pointing accuracy of 0.1° or better for Multispectral Imager | 0.10 | 0.0 | marginal |
| REQ-DATA-001 | The communication system shall support downlink of Multispectral Imager data at 100.0 Mbps | 2.00 | 20.0 | marginal |
| REQ-PWR-001 | The EPS shall provide positive power margin in all operating modes including eclipse | 70.90 | 100.0 | compliant |
| REQ-TTC-001 | The communication link shall close with at least 3 dB margin at minimum elevation | 7.70 | 156.7 | compliant |
| REQ-SYS-004 | The spacecraft shall be capable of deorbiting within 25 years of end-of-mission per space | 71.69 | 100.0 | compliant |

Total 8 requirements: 5 compliant · 3 marginal · 0 non-compliant (62.5%).

# 16. Technology Readiness

| Subsystem | Baseline (TRL) | Innovation (TRL) | Recommendation |
| --- | --- | --- | --- |
| eps | Triple-junction GaAs (Azur Space 3G30C) (9) | Perovskite-silicon tandem solar cells (5) | baseline |
| eps | Li-ion battery (SAFT VES16) (9) | Solid-state lithium battery (4) | carry_both |
| aocs | Reaction wheels + star tracker (9) | MEMS gyroscope + AI-based attitude determination (5) | carry_both |
| ttc | X-band transponder (conventional) (9) | Optical inter-satellite link (OISL) terminal (6) | baseline |
| propulsion | Monoprop hydrazine (9) | Water electrolysis green propulsion (5) | baseline |
| propulsion | Hall-effect thruster (8) | Electrospray (ionic liquid) thruster (6) | carry_both |
| structure | Aluminium honeycomb panels (9) | 3D-printed titanium lattice structure (6) | carry_both |
| thermal | MLI + radiator panels (9) | Phase-change material thermal buffer (5) | baseline |
| obdh | RAD750 / GR740 radiation-hardened processor (9) | COTS RISC-V with radiation-tolerant design (5) | baseline |

# 17. Reliability & FMECA

### Rationale

Series reliability model R_sys=∏R_i.  Over 3.0 yr (26298 h) the mission reliability is 0.718 against a target of 0.90.  Weakest contributor: aocs (R=0.924).  System MTBF is 60241 h.

### Assumptions

Constant failure-rate (exponential) model R(t) = e^{-λt}.

Series model: all subsystems must function; no parallel redundancy modelled here.

Failure rates from heritage (SMAD4 Table 19-5, MIL-HDBK-217F).

### Warnings & open issues

Mission reliability 0.718 < target 0.90; weakest: aocs (R=0.924). Consider redundancy.

Single-point failures: eps (R=0.936), aocs (R=0.924), obdh (R=0.949), payload (R=0.924)

## 17.1 Per-subsystem failure rates

| Subsystem | λ (1/h) | R(mission) | Redundancy |
| --- | --- | --- | --- |
| EPS | 2.50e-06 | 0.9364 | single string |
| AOCS | 3.00e-06 | 0.9241 | single string |
| TTC | 1.50e-06 | 0.9613 | single string |
| OBDH | 2.00e-06 | 0.9488 | single string |
| TCS | 5.00e-07 | 0.9869 | single string |
| PROPULSION | 4.00e-06 | 1.0000 | single string |
| STRUCTURE | 1.00e-07 | 0.9974 | single string |
| PAYLOAD | 3.00e-06 | 0.9241 | single string |

## 17.2 FMECA (top items)

| Item | Failure mode | Effect | S | O | D | RPN | Mitigation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EPS | Battery cell short | Loss of bus voltage; depleted battery | 4 | 3 | 2 | 24 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |
| AOCS | Reaction-wheel bearing seizure | Loss of fine pointing; switch to coarse mode | 4 | 3 | 2 | 24 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |
| OBDH | OBC SEU latch-up | Reset cycle; loss of telemetry window | 4 | 3 | 2 | 24 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |
| PAYLOAD | Detector dark current rise | Image SNR degradation; calibration drift | 4 | 3 | 2 | 24 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |
| TTC | Transmitter PA failure | Loss of downlink; use backup transmitter | 3 | 2 | 2 | 12 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |
| TCS | Heater string open | Component temperature excursion below limit | 3 | 2 | 2 | 12 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |
| STRUCTURE | Bolt back-out | Loss of deployable, latched structure | 3 | 2 | 2 | 12 | Cold/hot redundancy; safe-mode handoff; ground-commanded res |

# 18. Risk Register

Figure 12 — 5×5 risk index map (ECSS-M-ST-80C).

Risk register pending — no risks identified at this stage.

# 19. Space Debris Compliance

### Rationale

Post-mission disposal: orbital lifetime 2.7 yr with deorbit method 'natural'.  Casualty risk Ec=1.10e-08 (compliant against 1e-4 threshold).  Passivation score 1.00 covers stored-energy depletion of battery and propellant tanks per ISO 24113.  25-yr rule satisfied; 5-yr rule satisfied.

### Assumptions

Atmospheric drag lifetime per NRLMSISE-00 / JR-1971 (mean solar flux).

Casualty risk per NASA-STD-8719.14 demise model.

Passivation score weighted by battery + propellant tank stored energy.

Collision-avoidance ΔV budget assumed ~1 m/s per year in LEO.

| Parameter | Value |
| --- | --- |
| Orbital lifetime (yr) | 2.7 |
| 25-yr rule (IADC) | Compliant |
| 5-yr rule (FCC) | Compliant |
| Casualty risk Ec | 1.10e-08 |
| Casualty risk compliance | Compliant |
| Passivation score | 1.00 |
| Compliance score (0–100) | 92 |
| Deorbit method | natural |
| CA ΔV per year (m/s) | 2.5 |

# 20. Cost & Schedule

### Rationale

Parametric life-cycle cost estimate using the CubeSat COTS model. Hardware 151 kEUR + AIT 18 kEUR + NRE 45 kEUR (30% of HW) + launch 200 kEUR + operations 300 kEUR over 3.0 yr.  Total 0.7 MEUR with ±40% confidence interval at Phase A.

### Assumptions

Spacecraft class 'nano' → COTS pricing table.

AIT 12% of HW; NRE 30% of HW; ops €100k/yr.

Launch cost from rideshare/dedicated-launcher market reference.

All costs in 2025-EUR; no inflation escalation applied.

Phase-A P-curve: P50≈total, P70≈+15%, P80≈+25%, P90≈+50% per ECSS-M-ST-60C cost-uncertainty practice.

Figure 13 — Cost estimate at P50/P70/P80/P90 confidence levels.

Figure 14 — Cost breakdown by WBS element (DDT&E + recurring).

| WBS | Element | DDT&E (MEUR) | Recurring (MEUR) | Total (MEUR) |
| --- | --- | --- | --- | --- |
| 1.1 | Spacecraft hardware | 0.03 | 0.15 | 0.18 |
| 1.2 | AIT | 0.01 | 0.02 | 0.03 |
| 1.3 | Ground segment & ops | 0.00 | 0.30 | 0.30 |
| 1.4 | Launch service | 0.00 | 0.20 | 0.20 |

# 21. Design Loop Convergence

| Metric | Value |
| --- | --- |
| Converged | Yes |
| Iterations | 16 |
| Wall-clock time (s) | 0.02 |

# 22. Conclusions & Recommendations

The PDR baseline is now established.  Key engineering observations follow.

## 22.1 Open warnings

Link margin -2.6 dB < 3 dB minimum (required rate 100.0 Mbps vs max 54.4 Mbps)

Radiator area 0.11m² exceeds 50% of S/C area — consider active cooling

Mission reliability 0.718 < target 0.90; weakest: aocs (R=0.924). Consider redundancy.

Single-point failures: eps (R=0.936), aocs (R=0.924), obdh (R=0.949), payload (R=0.924)

## 22.2 Recommendations

[INNOVATION] STRUCTURE: Consider 3D-printed titanium lattice structure (TRL 6) as tech demo alongside baseline Aluminium honeycomb panels. Benefit: 30% mass reduction, integrated thermal paths. Risk: Qualification for flight loads ongoing

[INNOVATION] PROPULSION: Consider Electrospray (ionic liquid) thruster (TRL 6) as tech demo alongside baseline Hall-effect thruster. Benefit: Ultra-precise thrust, no moving parts. Risk: Limited total impulse for large dV

## 22.3 Next steps

Freeze the preliminary design and equipment selection.

Initiate engineering model build and breadboard activities.

Finalise the Verification Plan and Test Programme.